SAMPLER, SAMPLING AND EQUIPMENT SPECIFICATION

SECTION ONE

TPAO has determined standards and technical specifications for sampling and sampling equipment during drilling, WO, production etc. operations. It is essential that these standards are applied to all wells drilled for TPAO (including TPAO partnerships). It is the responsibility of the contractor to send the sample to the relevant unit upon TPAO request and no additional payment is made to the Contractor for this process.

Sample Types (including but not limited to the following);

- Cutting sample: Rock fragments that come to the surface during drilling are called cuttings. There are variations in the way these cuttings are taken for sampling purposes.

  - Tray sample: This is the sample taken to avoid missing any lithology intervals when the outcrop samples come in very large quantities.

  - Bag sample: Cutting samples placed by TPAO in a standardized bag for archiving.

    - Unwashed bag sample: Unwashed cutting samples placed in a bag with standards set by TPAO.

  - Envelope sample: Dried cutting samples placed by TPAO in standardized envelopes for archiving.

  - Plate sample: These are the cuttings samples that are placed in a standardized plate by TPAO for the examination of the Well Geologist.

  - Geochemistry sample: Unwashed cuttings samples taken by TPAO in a standardized canister or isojar for geochemical examination.

- Sieve sample: Cuttings samples placed in bags, plates or envelopes taken when drilling without sieve is required within the knowledge of the Well Geologist.

- Sample of leakage and lubricant material: In case of leakage; samples taken from the anti-leakage materials or lubricating oils and petroleum to be used.

- Core sample: These are the samples taken as a result of the coring operation, placed in trays without washing and labeled.

- Test sample DST (Drill Stem Test), Swab and Gas taken and labeled as a result of test operations

samples.

- Pre-log mud sample: It is a mud sample taken before the outflow for the logging operation to compare it with the fluid coming from the formation afterwards.

SECTION TWO

Sample Collection for Water Based Drilling Fluids

- Cutting Sample

- In exploration and appraisal wells, a cuttings sample will be taken every 2 m. In production wells, a cuttings sample will be taken every 10 m until 100 m before the expected target formation and every 2 m thereafter.

- Samples that are too soft or too fragile to be washed will be taken unwashed. Apart from these intervals, samples will be taken at the end of circulation and at depths deemed appropriate by the Well Geologist. If the amount of cuttings coming to the surface does not allow sieving (interruptions overflow the sieve for 2 m and sample loss occurs), the cutting sample should be taken using a tray.

Sample Tray

The cut tray must be cleaned after sampling

The sample that best represents the interval is taken by cutting the interruption mound in half.

Spot sampling (Spot / Hot Shot) should be done by taking a spoonful of cuts from fine and coarse sieves.

- The tray placed at the sampling point of the mud sieve is set aside after 2 m of sample accumulation. Cut the cut mound in half so that it fits into the washing sieve.

- Cuttings samples will be washed on 4-20-80 mesh sieves according to size. Two sieves with different mesh sizes are placed on top of each other according to the amount and diameter of the cuttings on the surface and placed at the appropriate point in the mud sieve.

Cutting samples are classified according to the way they are taken and stored;

- Bag Samples:

The fractions washed in the sieves will be placed in the sample bag. The bag sample will be oven dried.

- Envelope Samples

Of the cuts dried in the sample bag, one will be placed in envelopes to be archived for the R&D Center Department, one for the Regional Geology Directorate and one for MAPEG.

- Plate Samples

- The sieve washed sample will be placed wet on a plate for the Well Geologist to examine and describe under a binocular microscope. The Well Geologist may increase the number and intervals of plate samples if desired.

- The sample tray and/or sampling sieve must be manufactured in accordance with the structure of the mud sieve.

While preparing the samples accumulated in the sieves, no intervention with any material other than water.

- When the sampling depth is reached, the sample tray and/or sieves are removed and the spare ones are placed in front of the mud sieve without wasting time. After washing the sample with water, the two sieves are separated and the bag and plate samples are taken from the bottom sieve.

- When sampling, the sample is taken from the bottom with the sampling shovel as if cutting a pie slice in order to fully describe the 2 m drilling interval.

- Samples remaining in the large mesh sieve will be plate sampled when requested by the Well Geologist for debris control and delineation.

- The first sample is placed in the sample plate for the Well Geologist to examine.

- The cuttings in the sample tray are washed under unpressurized water in the sampler shed or in the Mud Logging Unit. 
The mud is removed as much as possible and the depth label is placed after the sample plate has been cut so that it is not overfilled.

- After the sample plate is filled with cut samples, the sample bag is filled.

- While the Possum Belly is being cleaned, the sample tray and/or sample sieves are removed from under the mud sieve. When the cleaning is finished, the mud sieve is washed and the sample tray and/or sieves are placed again.

- The samples taken in bags are dried in a drying oven. (90-100 C)

- One of the dried samples is placed in envelopes to be archived for the R&D Center Department, one for the Regional Geology Directorate and one for MAPEG.

- The prepared bag and envelope samples are placed in special boxes for archiving. The most important thing to note is that the samples should not be placed in the boxes wet.

Unwashed Bag Sample

Unwashed bag samples will be taken at intervals as requested by TPAO and its representative Well Geologist. These samples will be taken by placing nylon of the same size inside the sample bags to ensure waterproofing. Nylog bag will be heat sealed or sealed with a proper and safe method.

Geochemistry Sample:

Geochemistry samples will be taken at intervals and frequencies requested by TPAO. For geochemistry sampling, the cuttings are taken unwashed through sieves into cans or isojars with a volume of approximately 1 liter.

After 1/3 of the cut is placed in the box, 1/3 of pure water is added and 3-4 drops of zephiran are added for sterilization.

The box lid is tightly closed and sealed with silicone to ensure airtightness.

After the silicone has dried, turn the box upside down. After the box is turned upside down, it should not tip over or turn upside down and should be delivered to the R&D Center Department in this way.

- Sub- Sieve Sample:

When drilling without a sieve is required, the Well Geologist is informed. Continue sampling from the bottom of the sieve. It is very important to clean the bottom of the sieve after the sample has been taken from the respective meter.

- Leakage and Lubricant Material Sample:

In case of leakage; the anti-leakage materials, lubricants and oil to be used are first notified to the Well Geologist. After the necessary permission is granted by TPAO, two samples will be taken from the anti-leakage and lubricant materials used and one sample will be sent to the Regional Geology Directorate and one sample will be sent to the R&D Center Department upon request.

- Core Sample:

At the request of TPAO and/or where deemed appropriate by the Well Geologist, core samples are taken by coring operation. The paths to be followed in the core operation are as follows:

- During core drilling, a plate cut sample is taken at one meter intervals.

- If there is not sufficient recovery after core drilling, cuttings samples are taken at two meter intervals during expansion drilling.

- The start and completion of core drilling must be within the knowledge of the Well Geologist.

- It will be ensured that the cores are taken as  undamaged as possible and placed in 1 meter long trays.

- Placement in the trays will be done as shown in the figure below. The first core sample taken is placed in the tray numbered one, i.e. "M".

> Cores will be placed in roll nylons suitable for their diameter without washing and placed in trays.

Arrows indicating the drilling direction (drill) on the tray will be marked on the side of the tray.

- Core labels will be filled and affixed to the trays.

- After the cores are described in the well, the trays will be closed and sent to the R&D Center Department as soon as possible.

- Test Samples

- DST (Drill Stem Test) specimen:

DST operation is performed at the levels deemed appropriate by the Well Geologist and/or requested by TPAO.

The steps to be followed in the DST operation are as follows:

- For DST, mud samples will be taken from the pre-outlet circulation and the measurement values will be written in the report.

- In DST recovery, recovery samples will be taken and placed in appropriate containers under the supervision of the Well Geologist from the first recovery. Cards showing the DST information, the stand the sample came from and its number will be attached to the relevant samples. In case of recovery by reverse circulation, samples will be taken at intervals requested by the Well Geologist.

- After recovery is completed, the samples will be sent to the Reservoir Laboratory (or laboratories) requested by TPAO for analysis.

> If the recovery in exploration wells is petroleum, a sample is immediately sent to the Geochemistry Directorate of the R&D Center Department.

- Swab sample:

Swab samples will be taken from appropriate places to reflect the characteristics of the incoming fluid and changes and will be sent immediately to the laboratory requested by TPAO for analysis.

- Gas sample

Gas samples can be taken from the separator during workover or production testing in gas wells. If the required steel tubes cannot be supplied by TPAO, they shall be supplied by the Contractor upon request of TPAO. Sampling should be done by competent personnel.

- During sampling, the gas is allowed to flow out of the tube for a while so that the tube is completely filled with gas. When it is concluded that there is no air in the cylinder and it is completely gas coming from the well, the outlet valve is closed well and filled. Then the inlet valve is also closed. Check whether the valves are leaking or not by immersing the tube in water.

In order to examine the gases coming from the well in more detail during drilling, gas samples should be taken for analysis at the required levels. For this system, an Isotube gas sampling manifold is installed in the Mud Logging Unit.

- The Isotube allows gas sampling by simply plugging and unplugging and turning the valve control lever.

- The most important thing to note here is to insert the new tube immediately after the sample is taken. Since the sample line passes through the isotube, the system cannot read the gas if the new cylinder is not installed.

- Pre-Log Mud Sample

For log, mud sample is taken from the last circulation before outlet. Mud properties are measured, water loss is connected. Mud parameters and mud cake are delivered to the Log Engineer for logging. In full leakage wells, the pre-log mud sample is taken from the tank where the mud is pumped into the well.

Sample Collection for Oil Based Drilling Fluids

TPOA may decide to use petroleum or synthetic-based mud in specific sections or the entire well as required. When petroleum or synthetic-based mud is used, the collection of cuttings samples at the surface should continue regardless of whether MLU (Mud Logging Unit) is used or not. Due to the chemical nature of the drilling fluid used, personnel responsible for collecting, cleaning, and packaging the samples must use the necessary personal protective equipment (PPE) meticulously.

While this document generally outlines personal protective equipment and occupational health and safety topics, specific guidelines and rules in this regard should be implemented according to the contractor’s internal policies and contractual agreements.

- Key Considerations for Petroleum (or Synthetic) Based Sampling Procedures:

- Personnel responsible for sample collection and washing should not begin work without proper PPE to protect against petroleum (or synthetic) based fluids and potential gases.

  - In addition to mandatory PPE for drilling operations, the following protective gear should be worn in the specified order:

    - Protective (barrier) cream for potential skin contact

    - Latex or nitrile gloves suitable for the mud being used

    - Full-body coveralls with chemical-resistant hoods (e.g., Tyvek suits)

    - If coveralls do not protect the boots, chemical-resistant covers worn over boots

    - Long-sleeved chemical-resistant gloves

    - Chemical-resistant goggles

    - Respirator masks for protection against chemical and HC vapors

- Prevent water mixing with the mud system in the derrick; wash shakers, screens and sample trays with petroleum-based solvents (e.g., diesel).

- Do not use water guns or hoses in the shaker area.

- The initial washing of collected samples should be done in a barrel or large container within the shaker area using petroleum-based solvents (e.g., diesel).

- Petrol (or synthetic) based liquids and mud should not be transported to the MLU cabin or sample shack. Only samples that have undergone their initial washing should be moved to areas other than the shakers section.

- Samples that have undergone their initial washing should be re-washed in the sample shack using a detergent known to work well against the used petrol (or synthetic) based mud (e.g., dishwashing detergent).

- Even after being washed with detergent and thoroughly rinsed with water, samples should be dried in an oven at very low temperatures or at room temperature to prevent the release of various hydrocarbon gases.

- High-temperature drying methods such as stoves or other drying systems should definitely not be used.

  - Due to previous incidents of fire accidents caused by high-temperature sample drying, this matter is crucial for the safety of personnel.

- In cases where a large number of samples need to be collected and rapid drying is necessary, it is recommended to use specialized equipment or systems such as centrifuges or low-temperature drying.

- Regarding the “unwashed bag sample” and “geochemistry” samples mentioned in the previous section, when collecting samples for drilling operations using petrol (or synthetic) based mud, they should be carefully evaluated due to the risk of gas entrapment, explosion, and ignition. If there is a request for collection, they should be packaged and transported in accordance with hazardous material transportation rules. Considering the necessity of additional precautions, packaging the samples as washed samples would be one of the risk-reducing factors, especially when agreed upon with the Well Geologist.

If these guidelines are followed, the sampling methods and rules mentioned in the previous section remain the same for drilling operations using petrol (or synthetic) based mud.

SECTION THREE

Responsibilities in Sampling

- Whether or not MLU (Mud Logging Unit) is used during well operations (drilling, WO, production and all other operations), it is the Contractor's responsibility to provide the necessary infrastructure and technical support for proper and HSE regulated sampling and to carry out these actions with personnel and equipment.

- In cases where samples cannot be taken due to technical, HSE and infrastructural insufficiency, drilling will not continue. Once the sample has been collected, the operation can continue.

- The Contractor shall appoint a Sampler whose sole duty is to take samples in accordance with the principles set out in this document and to carry out other works described in this document. For 24 hours, there will be a sampler on each shift.

- Duties and Responsibilities of the Sampler:

- Obeying the HSE rules that are mandatory at the rig site.

- Taking samples from the wells in the specified quantities, washing them, making them conform to the standards determined for the examination of the Well Geologist, bagging, drying and making them ready for transportation when requested,

- To inform the Well Geologist when necessary during sampling at the wellhead,

- Noting the starting and ending depths of the samples taken at shift changes on the chart,

- To monitor the operation during DST in accordance with the direction of the Well Geologist, to take samples from the locations specified by the Well Geologist during test recovery and to make them transportable,

- Ensuring smooth casing of the core during core recovery,

- To fulfill the common duties and responsibilities determined for employees, to keep the workplace and geology laboratory clean,

- To perform all the above-mentioned works completely and duly, as well as other works to be assigned by the Well Geologist and to be responsible to the Well Geologist.

- A sampler assigned to a well may be assigned to other work or to another rig is subject to the approval of the Well Geologist in charge of the existing well.

- If requested by TPAO, the existing sampler will be replaced by a qualified person suitable for the job.

SECTION FOUR

Sample Shipping, Packaging and Other Consumables

- The Contractor is obliged to send all kinds of samples (envelope, bag, tube, isotube, core, DST, gas, swab, geochemistry, mud-petroleum sample, etc.) taken during drilling, workover and production tests to Regional Directorates, TPAO field laboratories, General Directorate-R&D Center Presidency and/or other addresses to be notified by TPAO by safe and fast transportation means, free of charge.

- The Contractor shall prepare and send the envelope samples taken during drilling as standard every 200 m for Exploration and Detection wells and at intervals to be determined by TPAO for Production wells. In addition, in cases where urgent sample examination is required, upon the request of TPAO, a sample will be sent to the specified addresses on the same day regardless of the quantity.

- The Contractor is obliged to supply all consumables required for sampling in drilling, workover and production tests free of charge (drums, test labels, canisters for geochemistry sample, core boxes, nylon and labels for core, sample bags, plastic boxes for sample bags and envelopes, cardboard boxes for packaging plastic boxes in which envelope samples are placed, sample plates, envelopes, pens, silicon, sieves, all kinds of chemicals used in sampling and TPAO's requests related to sampling and laboratory, etc.). materials).

- The Contractor shall prepare a hut-type Geology laboratory and have all kinds of materials ready in the laboratory before the drilling starts (tweezers, sample and sample looking plates, multi-core porcelain plates, strip meters, glass plates, industrial sample drying ovens, sample bags, necessary stationery materials, chemicals (acid, alizarin, phenolphthalein, acetone, etc.) plastic boxes for bag and envelope samples, cardboard boxes for packaging plastic boxes where envelope samples are placed and other materials requested by TPAO). Sample drying ovens will be of a size and quality to meet the needs during rapid drilling. Any inadequate and poor quality fixtures or consumables shall be replaced immediately upon request of TPAO.

- The technical requirements for the envelope, plastic sample box, sample containers, drying oven to be supplied are attached to and form part of this specification. All consumables and/or equipment to be supplied shall be provided with the approval of TPAO. Unsuitable materials and/or equipment will be replaced immediately.

- The laboratory to be prepared by the Contractor shall have the size, isolation, equipment and furnishing to provide an environment where the Well Geologist can work in a healthy manner. The fixtures of this laboratory such as tables, ergonomic chairs, armchairs, etc. will be complete and well maintained. Air conditioning and heating equipment will be ready for use in the laboratory and electricity and water connections will be made. Cleaning of the environment will be performed at regular intervals during the day and when requested by the Well Geologist.

- High speed internet connection with portable modem for use by the Borehole Geologist in the geology laboratory and/or office shed will be provided by the Contractor.

- TPAO reserves the right to suspend operations if the Contractor fails to take samples during operations due to lack of materials, lack of personnel, lack of water, etc.

- In operations where petroleum-based (or synthetic) mud is used, the Contractor shall provide all necessary additional consumables, equipment, and personal protective gear for sample collection at no additional cost. This includes ensuring that the samples are transported in accordance with regulations and rules.

ANNEXES

Annex: 1 Sample Envelope Specifications

The sample envelope must contain the following specifications;

- The sample envelope will be 8X14 in size,

- Paper quality will be 200 gram fancy paper and brown.

- The top part of the envelope should be foldable as shown in the attached figure and sample, and 1 cm from the top should be made of flexible, unbreakable, bendable metal material.

Annex-2 DST and Swab Sample Containers Specifications

Wire part

Wire part

Adhesive part

BACK FACE

FRONT FACE

Annex-3 Geochemistry sample container (Isojar)

| Specifications (There is a flexibility room in other dimensions except volume) |  |
| --- | --- |
| Volume (ml) | 1000 |
| Cover Diameter | 55mm |
| Inlet Diameter | 45mm |
| Width | 80mm |
| Height | 205mm |
| Empty bottle weight | 80 gr |
| Cover weight | 12 gr |

ISOJAR BOX

390

A-DETAIL

MATERIAL: PLASTIC BOX COLOR: WHITE

|  |  | DATE | NAME | SIGNATURE | T.P.A.O. GN. MAN. - - |  |
| --- | --- | --- | --- | --- | --- | --- |
| drawer |  | 01 06.2010 | P. SARIDAS |  |  |  |
| CHECKED BY |  | 15 06.2010 | A. ERGUN |  |  |  |
| APPROVED BY |  | 06/15/2010 | S. ERBEK |  |  |  |
| SCALE | SEDIMENTOLOGY GEO DIRECTORATE PLASTIC SAMPLE BOX ( SMALL SIZE 40x13x10 ) |  |  |  | STOCK NO | 79010496 |
|  |  |  |  |  | PART NO | — |
| 1/3 |  |  |  |  |  |  |
|  |  |  |  |  | PICTURE NO | 5664 |
|  |  |  |  |  | -- |  |

GENERAL TOLERANCE tO.2

NOTE : PRODUCTION WILL BE MADE ACCORDING TO SAMPLES

THICKNESS: 2.5mm

A-VIEW

MATERIAL NAME: 15x20CM TYREK (WATERPROOF FABRIC) INLET SHRINKABLE SAMPLE BAG

BASIC TECHNICAL SPECIFICATIONS:

- Sample bags shall be made of waterproof, heat resistant fabric.

- The finished carpet will be 15x20 cm in size.

- The inlet will be manufactured in a shrinkable way.

- The drawstring must be made of a type of thread (fine twine) that is not deformed and does not split at the ends.

- "TPAO LOGO" shall be printed on one side of each sample bag.

| Temperature Operating Adjustment Range | 70°C / 250°C |  |  |  |
| --- | --- | --- | --- | --- |
| Temperature Sensor | Fe-Const |  |  |  |
| Temperature Setting and Reading Accuracy | 1°C |  |  |  |
| Temperature Change (150°C)* | ± 2°C |  | ± 4°C |  |
| Temperature Fluctuation (150°C)* | ± 1°C |  | ± 2°C |  |
| Programmable Alarm Limits | ± 2°C / ± 10°C |  |  |  |
| Timer | 1 minute - 99.9 hours / Indefinite |  |  |  |
| Delayed Start Timer | 1 minute - 99.9 hours |  |  |  |
| Cell Volume, liter | 40- 60 | 120-150 | 200-250 | 390-500 |
| Net Volume, liter | 40- 50 | 110-150 | 190-220 | 370-400 |
| Number of Shelves (standard/maximum), pcs | 2/7 | 2/10 | 2/7 | 2/12 |
| Inner Surface Structure | Stainless steel |  |  |  |
| External Surface Structure | Powder coated steel |  |  |  |
| Installed Power | 800-1000 W | 1600-2000 W | 3100-3300 W | 4100-4300 W |
| Operating Voltage | 230 V - 50 Hz |  | 400 V - 50 Hz |  |

Annex-7: Logo:

For envelope-bag-box and other equipment https://www.tpao.gov.tr/logo-kullanimi the logo on the website and the example given below will be used. Different logos may also be used upon the request of TPAO