ANNEX "H"

# RIG AND STRING MATERIALS INSPECTION

# Each drilling rig to be provided by the Contractor shall be inspected according to API RP 4G, 3rd edition April 2004, Category IV. The inspection report shall be made by API or Turkak agency certified companies acceptable by the Operator. Inspection of drill string components must be carried out by accredited companies acceptable to the Operator on the basis of API RP 7G-2, Table E,1 standards Inspection reports prepared for rig and string elements shall be submitted to the Operator before spud. After a certain period of use of the drill string, routine inspection of the drill string at the intervals specified in the table must be carried out by the Contractor. If an unsafe situation is detected by the representative of the Operator, the drilling rig and string material will be inspected (visual, ultrasonic, magnetic particle inspection, etc. non-destructive inspection) at the expense of the Contractor. There is no additional charge for the inspection by the Operator, it is included in the bid price.

# Records of all materials and equipment used in the rig, from the first time they are installed and started to be used, will be made available in the rig.

| Type of material | Inspection time |
| --- | --- |
| Drill Pipe, HWDP, subs | At the start of work and after every 1500 drilling hours (rotating hour) |
| DC, Stabilizer | At the start of the job and after every 300* drilling hours (rotating hour) |

# * In the 200 Ton Capacity rig, DC, Stabilizer inspection time is at the start of work and every 400 drilling hours.

1-Drill pipes and HWDPs are inspected to cover the following items.

(A) magnetic particle inspection (MPI)

(B) ultrasonic inspection;

(C) Internal and external visual inspection

(D) outer diameter measurement

(E) tool joint and thread inspection

2-Drill collars, Kelly, lifting sub, adaptor sub, all handling toolar etc. are inspected to cover the following items.

(A) pin and box inspection

(B) pin and box magnetic particle inspection (MPI)

3-Stabilizer is inspected to cover the following items. Outer Diameter measurement of the Stabilizers will be performed at each full exit maneuver, the thread inspection will be performed simultaneously with the DCs.

- Thread inspection

- Magnetic particle inspection (MPI) for pin and box

- Outside diameter measurement (1/8" diameter up to drop is acceptable)

4- Rig inspection report will be submitted to the Operator before the spud.

- Performed according to API RP 4G, 3rd edition, April 2004, Category IV

- The date of the inspection report must not exceed 4 years at the date of tender.

5- Draw works drum and brake hoops, slings, drill ropes and all lifting ropes, Top Drive, travelling blocky and hook, crown block, dead-line anchors, drill pipe and casing elevators, elevator link arms, casing lowering equipment (slips, spiders) and safety clamps inspection shall be carried out by the Contractor in the period between the contract signature date and the spud and the inspection report shall be submitted to the Operator before the spud. After the first spud, this inspection process is carried out again at every process between demob and mob, at least every 6 months for rigs with a minimum capacity of 200 tons, and the inspection report is submitted to the Operator. For the rigs with a minimum capacity of 300 tons, this inspection process is repeated at the end of each well and the inspection report is submitted to the Operator.

- Inspection will be performed according to API RP 8B,8th edition, May 2014, Category III (NDT-Nondestructive).