1 ¼” COILED TUBING NITROGEN CLEANOUT SERVICES

- Scope of Work

Company wishes to receive Coiled Tubing Services which includes wellbore cleanout and N2 Lifting in TPAO’s oil and gas wells located in Thrace region, but not limited to that region. Subject to the well conditions encountered, Contractor shall carry out N2 assisted well flow and wellbore cleanout such as debris, scale and other blockages removal. Service period shall be two (2) years from the date of first signed.  Contractor shall provide the necessary operations and engineering capability together with computer facilities and software to deliver:

·         Well intervention modelling for running weights, pulling weights as required

·         Tool string proposals

·         Rig up schematics

·         Coiled Tubing Life / Fatigue Real Time, Op. Envelope

- Coiled Tubing Personnel and Equipment Technical Requirements

- Coiled Tubing Control Cabin

Shall contain all necessary controls, communication and calibrated monitoring facilities to ensure a safe and efficient operation.

- Coiled Tubing Size and Properties

CT pipe OD size shall be applicable to work properly and efficiently inside the TPAO’s production and frac strings. Therefore, CT pipes OD should be 1 ¼” OD. CT pipe shall has burst strength of minimum 5K psi. Reel capacities shall be 3000 meters.

- Injector

Minimum 30000 lbs pull capacity

- Wellhead Equipment

- 2 9/16” 5K and 10K

- Combi or Quad BOP, 5000 or 10.000 psi

- Risers, 5000 psi or 10.000 psi

- Dual Hydraulic Stripper, 5.000 or 10.000 psi

- Flow Cross

- Necessary XO’s to the Frac head and TP wellhead equipment (2 9/16” API 5K or 10K Manuel Gate Valve)

- 4-1/16” 10K (Minimum)

- Combi or Quad BOP, Min. 10.000 psi

- Risers, Min. 10.000 psi

- Dual Hydraulic Stripper, Min. 10.000 psi

- Flow Cross

- Necessary XO’s to the Frac head and TP wellhead equipment (4.1/16” API Min. 10K Manuel Gate Valve)

- Pumping System

- Min. 450 HP per each engine always ready at one time

- 2 diesel driven pumps from the separate engine

- There shall be effective screen at the high pressure end to keep fluid clean

- Capable of pumping hot water and viscous or abrasive fluids

- Capable of pumping low rate, high pressure (0,3 bpm to 5 bpm at up to 10.000 psi)

  - Shall have adjustable relief valve fitted on unit and valve must be directed safely away from work area

- High Pressure Iron

- Shall be Figure 1502

- 2" ID

- Picker

CT Unit shall be supported by a Crane System with minimum 3 ton lifting capacity.

If the COMPANY requests extra crane, CONTRACTOR will provide it.

- BHA Tools

All necessary tools to properly and efficiently perform cleanout and nitrogen lifting operations. Bottom Hole Tools shall have compatible sizes to work efficiently and properly inside TPAO’s frac and production strings.

Consumables

All necessary consumables (methanol, liquid nitrogen etc.) to properly and efficiently perform cleanout and nitrogen lifting operations.

- Nitrogen Pumping System

- Minimum Flow rate 250 scf/m

- Minimum Working pressure 5.000 psi

- N2 tanks: Necessary capacity

- Minimum 450 HHP Nitrogen Pumper

- Nitrogen Membrane System

- Minimum flow rate 250 scf/m

- Minimum working pressure 3000 psi

- N2 minimum purity: 95 %

Personnel

Contractor shall provide necessary personnel for the scope of work herein.  All personnel shall be fully qualified and be able to inspect, adjust, prepare, assemble, redress, maintain and/or repair Contractor supplied equipment.

The following list shows the minimum requirements for the personnel to be supplied by the Contractor in order to carry out the Services in accordance with the scope of work herein.

| Personnel | Number of Shiftsper day | Min. number of personnel per shifts | Level of Experience |
| --- | --- | --- | --- |
| CT Engineer / Supervisor | Two | 1 | +5 years |
| CT Operator | Two | 1 | + 2 years |
| N2 Operator | Two | 1 | + 2 years |
| Helper | Two | TBD | + 1 years |

## HSE Policy

- CONTRACTOR should provide hard hat, safety glasses, and steel-toed boots when outside the living accommodations and during all operations and emergency situations for their own personnel.

- CONTRACTOR shall, as a minimum, brief all personnel engaged in the Work of all the details of the aforementioned requirements imposed on CONTRACTOR.

- Personal must be well trained according to the HSE policy and has to have necessary documents.

- CONTRACTOR shall keep the Work site free at all times from accumulation of water, material, or rubbish resulting from Work, and upon completion of Work shall thoroughly clean the premises and issue the removal of all Contractor Equipment.

- CONTRACTOR shall take all necessary precautions to comply with applicable regulations protecting environment.

- No trash shall be left on the well site.

- Responsibilities of Contractor

Contractor shall:

- After receiving call out e-mail from TPAO for Coiled Tubing Services, be ready for demanded operations within 48 hours if mobilization distance is less than 600 km, within 72 hours for the mobilization distance more than 600 km. For waits after the specified call-out time, a penalty is given as the rig hourly fee for each expected hour. This penalty can be up to 24 hours.

- Complete all customs procedures, if required.

- Provide qualified and experienced service engineers and technical personnel to support  Coiled Tubing Services as required.

- Provide inspection reports of all necessary iron requirements, treating manifolds and well head equipment.

- Submit pull test report showing that it does not fall below 90% of the maximum injector pulling capacity.

- Submit CVs for Key Personnel and case histories for Coiled Tubing Services which includes wellbore cleanout and N2 Lifting.

- Provide properly sized state of the art equipment (and back-ups) and instrumentation for Coiled Tubing Services. If necessary, nitrogen membrane generator system shall be activated within 3 months after mutual agreement is reached by parties.

- Provide recovery fluid tank (Min. 100 bbl).

- Carry out recovery fluid disposal and acid neutralization if needed.

- Provide diesel fuel and electricity for Contractor’s equipment, adequate lightning in the working area, manlift for wellhead and crane for loading and unloading Contractor’s material at the wellsite.

- Provide catering, potable water, housekeeping and one office shed on location for Company’s personnel up to 5 people and all Contractor’s personnel and personnel of its Subcontractors.

- Provide a toilet shed with portable septic tank for rigless operation.

- Supply water hauling service for Coiled Tubing Services.

- Undertake all mobilization of equipment from initial mobilization point of Contractor

to the Company’s well site and final demobilization from the well site.

- Provide a list of Contractor’s all equipment at the well site indicating status and quantity of each tool and equipment.

- Provide an engineer or a supervisor for each required Service to coordinate between Company and Contractor to issue the equipment and personnel timely manner, and to present technical expertise and support.

- Follow the instructions in Coiled Tubing Services program or provide an explanation of any deviation from the original program to the Company.

- Take all responsibility of Contractor’s personnel.

- Perform tool and equipment checks prior to the start of each operation. Complete the Contractor’s QA/QC forms.

- Check the operation location area and give feedback about any deficiencies of checked location before the operation.

- Provide the certificates for required tools and equipment to commence the work.

- Prepare wellsite equipment layout plan before mobilization of Contractor’s equipment.

- Provide daily reports to the Company Representative at the end of each day. Prepare field tickets and get approval of Company Representative. Provide technical reports and graphs as required.

- Notify Company Representative about anticipated problems.

- Fully participate in the daily morning meeting and all safety meetings at the well site.

- Wear Contractor’s uniform and the proper PPE required at Company’s well site. Contractor should warn Company about the safety of the working conditions, and take all safety precautions.

- Provide an end of service report in hard copy and in e-media.

- Provide an end of WORK report in e-media and hardcopy in accordance with the report format of COMPANY

- Ensure the equipment supplied to be fit for its intended purpose as per international standards and as outlined in this Specification.

- Provide extensive spares and fully equipped toolbox with the equipment.

- All lighting equipment in the operation area must be ex-proof.

- Special Terms and Conditions

- Contractor shall not subcontract of Coiled Tubing Services identified at Scope of Work section and shall not use third parties’ Coiled Tubing Unit, Pumping Unit and Nitrogen Pumping System.

- Company will not pay any Redress charge for Contractor’s equipment.

- Company will not pay any LIH pipe charge for periodical cut pipes.

- Company will not pay any crane / forklift charge for loading and unloading operations.

- Company will not pay any man basket or scaffholding charge for activities at height.

- Company will not provide any personnel, equipment, consumable or related material for performing of the work on site.  All related personnel, equipment and consumable material will be provided by Contractor.

- In the event that NPT (non productive time) is caused by failure of Contractor’s equipment, Company shall pay zero rate for the equipment and personnel for the duration of the NPT that is caused by Contractor’s equipment. The duration and proof of failure shall be documented and reported to both Contractor and Company by Company representative at the work site within 3 days of the occurrence of the failure.

- Contractor will have to use their proposed tools/ equipments and materials whose technical information correspond to Company’s requirements declared in tender documents.

- When the coil tubing will start to RIH, Personnel charges will start and continue until the written release notice from company representative.

- Daily Rental or operational charges for equipment shall commence when the coil tubing will start to RIH and shall be ceased when the Company releases the Contractor. Rental or operational charges will not be applied the day after the equipment are released.

- Bidder’s commercial quotation shall be prepared as unit price bases.

- During the duration of the Contract for Coiled Tubing Services, unit price stated in the bidder’s commercial quotation will be used.

- Personnel charges will be lump sum which means one (1) CT Supervisor or Engineer, two (2) Specialist or Operator and three (3) Helper in 12 hour shift will be ready to carry out N2 Lifting . Contractor shall provide two (2) 12 hour shift groups per day.

- Mobilization & Demobilization of the equipment for each operation will be accepted as one way transportation price. In case of unsatisfactory performance of Contractor’s equipment, Contractor shall replace such equipment at no cost (mobilization, demobilization, import- export etc.) to Company.

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

END OF THE COILED TUBING TECHNICAL SPECIFICATION